Index

Software Updates	1

#16.003 - It will not be possible to transfer parameter tables from the PC to the KEYCF	1

#14.625 - Device 20 disappears when quitting Windnc.	1

#15.085 - The new parameters of the 8055 do not have a comment associated with them in Windnc.	1

#15.357 - With an 8025 CNC, the Windnc would lock up when editing a program.	1

#16.856 - With an 8025 CNC, the tool file did not appear.	1

# Software Updates

## #16.003 - It will not be possible to transfer parameter tables from the PC to the KEYCF

## #14.625 - Device 20 disappears when quitting Windnc.

In SETUP, if 19 devices + LOCAL have been created and a new one is created, when getting out of Windnc and going back in, the new device has disappeared and the LOCAL one is selected.

## #15.085 - The new parameters of the 8055 do not have a comment associated with them in Windnc.

The new axis parameters APx and ALx (leadscrew error compensation) of version V12.xx of the 8055 CNC did not have a comment associated with them in Windnc.

## #15.357 - With an 8025 CNC, the Windnc would lock up when editing a program.

While editing a program at an 8025 CNC, when writing an open parenthesis and then anything else, the DNC would lock up.

## #16.856 - With an 8025 CNC, the tool file did not appear.

While working with an 8025 CNC, it was possible to download the parameters and the programs, but the tool table could not be downloaded.